Flight Test Date: 2/25/2026

Simulink Model: ground_test_EMF (same as HoverTest_OLHeave)

Set up script:

%% EMF controller

% --------------

%Initalize Controller

% --------------

modelpath = 'JohnnyRMACLinearModels.mat';

trimSpeed = 0; % m/s

[EMF, aircraftIC] = setupEMF(modelpath, trimSpeed);

% --------------

% Load Hover Linear Model

% States = [x y z phi theta psi u v w p q r]

% Outputs = [phi theta psi u v w p q r Omegas udot vdot wdot pdot qdot rdot]

% Inputs = [dcol dlat dlon dped]

% --------------

tmp = load(modelpath);

HoverModel.A = tmp.As(:,:,1);

HoverModel.A(12,8) = 0; % Zero out Nv - likely due to wonky drag modeling but need to check

HoverModel.B = tmp.Bs(:,:,1); % Includes pseudo inverse

HoverModel.C = [tmp.Cs(4:16,:,1); tmp.As(7:12,:,1)]; % Add accelerations to outpus

HoverModel.D = [tmp.Ds(4:16,:,1); tmp.Bs(7:12,:,1)];

% --------------

% Initalize Conditions

% --------------

InitCond.lat_rad = 0.6229;

InitCond.lon_rad = -1.8491;

InitCond.ze_m = -100; %Inital Altitude

InitCond.phi_rad = interp1(EMF.VTAB, tmp.x_trim(4,:), trimSpeed);

InitCond.theta_rad = interp1(EMF.VTAB, tmp.x_trim(5,:), trimSpeed);

InitCond.psi_rad = interp1(EMF.VTAB, tmp.x_trim(5,:), trimSpeed);

InitCond.u_mps = interp1(EMF.VTAB, tmp.x_trim(6,:), trimSpeed);

InitCond.v_mps = interp1(EMF.VTAB, tmp.x_trim(7,:), trimSpeed);

InitCond.w_mps = interp1(EMF.VTAB, tmp.x_trim(8,:), trimSpeed);

InitCond.Omega1_rps = interp1(EMF.VTAB, tmp.u_trim(1,:), trimSpeed);

InitCond.Omega2_rps = interp1(EMF.VTAB, tmp.u_trim(2,:), trimSpeed);

InitCond.Omega3_rps = interp1(EMF.VTAB, tmp.u_trim(3,:), trimSpeed);

InitCond.Omega4_rps = interp1(EMF.VTAB, tmp.u_trim(4,:), trimSpeed);

InitCond.IdleOmega_rps = 1750*pi/30;

% --------------

% Initailize for PWM signal

% --------------

Kv = 1750; % Motor Constant, RPM/V

MaxOm_rps = Kv*25*pi/30*0.81;

MinOm_rps = 0;

MaxPWM_ms = 2000;

MinPWM_ms = 1098; % This is increased to the minimum PWM when armed

% --------------

% Initailize for Tracking Response (integral anti-windup)

% --------------

Enable_TR = 1;

Ktr_Col = 0.3;

Ktr_Lat = 3;

Ktr_Lon = 1;

Ktr_Ped = 1;

% --------------

% ACAH Gains - Updated to match PID rate feedback gains as of 1/14/26

% --------------

MixerGain(1) = mean(abs(HoverModel.D(10:13,1)));

MixerGain(2) = mean(abs(HoverModel.D(10:13,2)));

MixerGain(3) = mean(abs(HoverModel.D(10:13,3)));

MixerGain(4) = mean(abs(HoverModel.D(10:13,4)));

EMF.ACAH.RollP = 15*MaxOm_rps/1000/MixerGain(2);

EMF.ACAH.RollI = EMF.ACAH.RollP*0.1;

EMF.ACAH.RollD = 30*MaxOm_rps/1000/MixerGain(2);

EMF.ACAH.PitchP = 15*MaxOm_rps/1000/MixerGain(3);

EMF.ACAH.PitchI = EMF.ACAH.PitchP*0.1;

EMF.ACAH.PitchD = 30*MaxOm_rps/1000/MixerGain(3);

EMF.ACAH.YawP = 30*MaxOm_rps/1000/MixerGain(4);

EMF.ACAH.YawI = 15*MaxOm_rps/1000/MixerGain(4);

Test Matrix:

| Task No. | Task Description |
| --- | --- |
| 0 | Initial test of the controller - Motor disarmed, rotating the drone by hand in pitch, roll, and yaw (positive rotation first followed by a negative rotation) – [50 100]s - Control stick doublet input in pitch, roll, and yaw axis while throttle < 40 % - [100 150]s - Attempt to take off with a pitch doublet command – [150 200]s |
| 1 | Multiple attempts to take off but the drone continued to tilt to one side. Large stick inputs are required to center the attitude of the drone. |
| 2 | Hand holding the drone while the feedback is on to test the compensation from the controller – [120 160]s |

A gain of 10 is placed to increase the control input in roll

A gain of 2 is placed to increase the control input in yaw

Flight Test Date: 2/25/2026

Simulink Model: ground_test_EMF

Test Matrix:

| Task No. | Task Description |
| --- | --- |
| 2 | Initial test of the controller - Motor disarmed. Drone is resting on solid ground– [115 150]s - Throttle at ~40 % - [160 185]s |
| 3 | Multiple attempts to take off but the drone continued to tilt to one side. Large stick inputs are required to center the attitude of the drone. |
| 4 | Hand holding the drone while the feedback is on to test the compensation from the controller - The drone is resting on level surface, hand tilting the drone in pitch, roll, and yaw axis – [250 293]s - Thrust raised to 50 % and holding it in the air, rotating it in pitch, roll, and yaw axis to feel the compensation of the controller. Roll and yaw axis much weaker than pitch axis – [300 372]s |

Test Script:

%% EMF controller

% --------------

%Initalize Controller

% --------------

modelpath = 'JohnnyRMACLinearModels.mat';

trimSpeed = 0; % m/s

[EMF, aircraftIC] = setupEMF(modelpath, trimSpeed);

% --------------

% Load Hover Linear Model

% States = [x y z phi theta psi u v w p q r]

% Outputs = [phi theta psi u v w p q r Omegas udot vdot wdot pdot qdot rdot]

% Inputs = [dcol dlat dlon dped]

% --------------

tmp = load(modelpath);

HoverModel.A = tmp.As(:,:,1);

HoverModel.A(12,8) = 0; % Zero out Nv - likely due to wonky drag modeling but need to check

HoverModel.B = tmp.Bs(:,:,1); % Includes pseudo inverse

HoverModel.C = [tmp.Cs(4:16,:,1); tmp.As(7:12,:,1)]; % Add accelerations to outpus

HoverModel.D = [tmp.Ds(4:16,:,1); tmp.Bs(7:12,:,1)];

% --------------

% Initalize Conditions

% --------------

InitCond.lat_rad = 0.6229;

InitCond.lon_rad = -1.8491;

InitCond.ze_m = -100; %Inital Altitude

InitCond.phi_rad = interp1(EMF.VTAB, tmp.x_trim(4,:), trimSpeed);

InitCond.theta_rad = interp1(EMF.VTAB, tmp.x_trim(5,:), trimSpeed);

InitCond.psi_rad = interp1(EMF.VTAB, tmp.x_trim(5,:), trimSpeed);

InitCond.u_mps = interp1(EMF.VTAB, tmp.x_trim(6,:), trimSpeed);

InitCond.v_mps = interp1(EMF.VTAB, tmp.x_trim(7,:), trimSpeed);

InitCond.w_mps = interp1(EMF.VTAB, tmp.x_trim(8,:), trimSpeed);

InitCond.Omega1_rps = interp1(EMF.VTAB, tmp.u_trim(1,:), trimSpeed);

InitCond.Omega2_rps = interp1(EMF.VTAB, tmp.u_trim(2,:), trimSpeed);

InitCond.Omega3_rps = interp1(EMF.VTAB, tmp.u_trim(3,:), trimSpeed);

InitCond.Omega4_rps = interp1(EMF.VTAB, tmp.u_trim(4,:), trimSpeed);

InitCond.IdleOmega_rps = 1750*pi/30;

% --------------

% Initailize for PWM signal

% --------------

Kv = 1750; % Motor Constant, RPM/V

MaxOm_rps = Kv*25*pi/30*0.81;

MinOm_rps = 0;

MaxPWM_ms = 2000;

MinPWM_ms = 1098; % This is increased to the minimum PWM when armed

% --------------

% Initailize for Tracking Response (integral anti-windup)

% --------------

Enable_TR = 1;

Ktr_Col = 0.3;

Ktr_Lat = 3;

Ktr_Lon = 0.5;

Ktr_Ped = 1;

% --------------

% ACAH Gains - Updated to match PID rate feedback gains as of 1/14/26

% --------------

MixerGain(1) = mean(abs(HoverModel.D(10:13,1)));

MixerGain(2) = mean(abs(HoverModel.D(10:13,2)));

MixerGain(3) = mean(abs(HoverModel.D(10:13,3)));

MixerGain(4) = mean(abs(HoverModel.D(10:13,4)));

EMF.ACAH.RollP = 15*MaxOm_rps/1000/MixerGain(2);

EMF.ACAH.RollI = EMF.ACAH.RollP*0.1;

EMF.ACAH.RollD = 30*MaxOm_rps/1000/MixerGain(2);

EMF.ACAH.PitchP = 15*MaxOm_rps/1000/MixerGain(3);

EMF.ACAH.PitchI = EMF.ACAH.PitchP*0.1;

EMF.ACAH.PitchD = 30*MaxOm_rps/1000/MixerGain(3);

EMF.ACAH.YawP = 30*MaxOm_rps/1000/MixerGain(4);

EMF.ACAH.YawI = 15*MaxOm_rps/1000/MixerGain(4);

% --------------

% RCHH Gains

% --------------

EMF.RCHH.P = 3;

EMF.RCHH.I = 1.5;

% Flags

EnableSpeedHold = 0;

EnableRCHH = 0;

EnableInputNoise = 0;